Tech-Web Client/Server API's

MIMOSA's Tech-Web API's are used for communicating data and information between condition monitoring systems, diagnostic systems, asset management systems, and work management systems via HTTP or HTTPS (secure) protocol using XML messages.  The API's are defined using XML Schema  and specified in a client/server fashion.

The API's are grouped into 8 application packages:

Trend-Web:	Allows retrieval of historical scalar measurements, abnormal data alarms, and operational event logs

For:	Process Data Historians

Process Condition Monitoring Systems

Operational Data Systems

Dyn-Web:	Allows retrieval of historical dynamic measurements (used with vibration and sound monitoring and including frequency spectra measurements and time waveforms), abnormal data alarms, and operational event logs

For:	Vibration Condition Monitoring Systems

Sound Condition Monitoring Systems

BLOB-Web:	Allows retrieval of historical binary large objects (BLOB) measurements (used with thermography and imaging monitoring), abnormal data alarms, and operational event logs

For:	Thermographic Condition Monitoring Systems

Image Monitoring Systems

Sample-Web:	Allows retrieval of historical fluid, air, and solid sampling data, abnormal data alarms, and operational event logs

For:	Oil Sampling Condition Monitoring Systems

Air Sampling Condition Monitoring Systems

Solid Sampling Condition Monitoring Systems

Reg-Web:	Allows retrieval of "as-designed" segment hierarchical breakdown of facility, process, and machine systems, along with the "as-installed" asset information.  Also allows access to  nameplate and image data on individual assets and models, including component part breakdowns.

For:	OEM Model Information Systems

Asset Registry Information Systems

Maintenance Management Systems

Piping & Instrumentation Design Systems

Diag-Web:	Allows retrieval of human or "smart-agent" generated current and/or future proposed asset health states, current and/or future proposed diagnostic failure modes and casual trees, remaining useful life predictions, and recommendations.  Also allows access to measurement evidence supporting the diagnoses/prognoses.

For:	Diagnostic Systems

Prognostic Systems

Work-Web:	Allows the creation and audit tracking of a new work request in a work management system for a service segment or a serialized asset.  Allows the retrieval of work orders and work order steps, and actual work completed information..  Also allows the retrieval of pre-planned work packages ("solution packages").

For:	Maintenance Management Systems