OSA-EAI Application Technology Packages

MIMOSA's OSA-EAI application technology packages are grouped into 8 application packages:

Trend-Web:	Allows retrieval of historical scalar measurements, abnormal data alarms, and operational event logs

For:	Process Data Historians

Process Condition Monitoring Systems

Operational Data Systems

Dyn-Web:	Allows retrieval of historical dynamic measurements (used with vibration and sound monitoring and including frequency spectra measurements and time waveforms), abnormal data alarms, and operational event logs

For:	Vibration Condition Monitoring Systems

Sound Condition Monitoring Systems

BLOB-Web:	Allows retrieval of historical binary large objects (BLOB) measurements (used with thermography and imaging monitoring), abnormal data alarms, and operational event logs

For:	Thermographic Condition Monitoring Systems

Image Monitoring Systems

Sample-Web:	Allows retrieval of historical fluid, air, and solid sampling data, abnormal data alarms, and operational event logs

For:	Oil Sampling Condition Monitoring Systems

Air Sampling Condition Monitoring Systems

Solid Sampling Condition Monitoring Systems

Reg-Web:	Allows retrieval of "as-designed" segment hierarchical breakdown of facility, process, and machine systems, along with the "as-installed" asset information.  Also allows access to  nameplate and image data on individual assets and models, including component part breakdowns.

For:	OEM Model Information Systems

Asset Registry Information Systems

Maintenance Management Systems

Piping & Instrumentation Design Systems

Rel-Web:	Allows retrieval of reliability study, FMECA analysis, failure analysis, and root cause information related to assets and "as-designed" segments.

For:	Reliability Management Systems

RCM Information Systems

FMECA Analysis Systems

Root Cause Analysis Systems

OEM Model Information Systems

Asset Registry Information Systems

Maintenance Management Systems

Diag-Web:	Allows retrieval of human or "smart-agent" generated current and/or future proposed asset health states, current and/or future proposed diagnostic failure modes and casual trees, remaining useful life predictions, and recommendations.  Also allows access to measurement evidence supporting the diagnoses/prognoses.

For:	Diagnostic Systems

Prognostic Systems

Work-Web:	Allows the creation and audit tracking of a new work request in a work management system for a service segment or a serialized asset.  Allows the retrieval of work orders and work order steps, and actual work completed information..  Also allows the retrieval of pre-planned work packages ("solution packages").

For:	Maintenance Management Systems