OSA-EAI Work-XML  V3.0 Server

Support Level 1 Interfaces

Revised:  March 8, 2004

### For All Tech-XML Servers

### Establish Connection

V3-0-0003-nnConnect.xsd 			(Allows clients to connect with a valid

connection string and establish a session

with the server.  Returns to the client a session identifier and the technologies the server supports – such as REG-XML, WORK-XML, & TREND-XML)

### Establish Disconnection

V3-0-0004-nnDisconnect.xsd 	(Allows a client to disconnect from the server.   Server will automatically perform this function if the client is not active for a server-defined timeout interval.)

### For Work-XML Servers, Read-only Access, Support Level 1

### Serve Work/Action Management Setup Info.

V3-0-8028-nnQueryPriorityType.xsd 		(Returns priority types)

V3-0-8029-nnQueryWorkManageType.xsd	(Returns work management types)

V3-0-8030-nnQueryWorkTaskType.xsd		(Returns a work task type)

V3-0-8039-nnQueryWorkAuditType.xsd		(Returns work status audit types)

V3-0-8003-nnQuerySolForAs.xsd	(Returns solution packages (preplans)

applicable to an Asset)

V3-0-8002-nnQuerySolForSg.xsd	(Returns solution packages (preplans)

applicable to a Segment)

### Serve Agent Info.

V3-0-8027-nnQueryAgent.xsd 			(Returns agents)

### Serve Asset-specific Work/Action Management Information

V3-0-8011-nnQueryAsRFW.xsd			(Returns Requests for Work related to an Asset)

V3-0-8025-nnQueryAsCompWork.xsd		(Returns Asset completed work history)

### Serve Segment-specific Work/Action Management Information

V3-0-8010-nnQuerySgRFW.xsd			(Returns Requests for Work related to a

Segment)

V3-0-8024-nnQuerySgCompWork.xsd		(Returns Segment completed work history)

### Serve General Work/Action Management Information

V3-0-8016-nnQueryWR.xsd			(Returns a Work Request)

V3-0-8023-nnQueryWOStepforWR.xsd		(Returns a Work Order Step for a Work

Request)

V3-0-8021-nnQueryWOStep.xsd	(Returns a Work Order Step)

V3-0-8019-nnQueryWO.xsd			(Returns a Work Order)

V3-0-8017-nnQueryWRAudit.xsd	(Returns the status of a Work Request)

V3-0-8022-nnQueryWOStepAudit.xsd	(Returns the status of a Work Order Step)

V3-0-8020-nnQueryWOAudit.xsd	(Returns the status of a Work Order)

### For Work-XML Servers, Write-only Access, Support Level 1

### Receive New Asset Request for Work and Generate New Work Request

V3-0-8007-nnCreateAsRFWandWR.xsd	(Creates an Asset Request for Work (RFW) and a Work Request (WR) structure from a client-supplied RFW structure.  The returned Work Request primary key will be utilized by the client to track the work submitted.)

### Receive New Segment Request for Work and Generate a New Work Request

V3-0-8009-nnCreateSgRFWandWR.xsd	(Creates Segment Request for Work (RFW) & Work Request (WR) from a submitted client RFW.  The returned Work Request will be utilized by the client to track the work submitted.)

### Receive New Binary Attachment to Work Request

V3-0-8031-nnCreateWRBlob.xsd	(Creates a Binary attachment to a Work Request.  This interface can be repeated, if the server supports multiple binary attachments.  A server may or may not allow binary attachments.

### For Work-XML Servers, Read/Write Access, Support Level 1

### Serve Work/Action Management Setup Info.

V3-0-8028-nnQueryPriorityType.xsd 		(Returns priority types)

V3-0-8029-nnQueryWorkManageType.xsd	(Returns work management types)

V3-0-8030-nnQueryWorkTaskType.xsd		(Returns a work task type)

V3-0-8039-nnQueryWorkAuditType.xsd		(Returns work status audit types)

V3-0-8003-nnQuerySolForAs.xsd	(Returns solution packages (preplans)

applicable to an Asset)

V3-0-8002-nnQuerySolForSg.xsd	(Returns solution packages (preplans)

applicable to a Segment)

### Serve Agent Info.

V3-0-8027-nnQueryAgent.xsd 			(Returns agents)

### Serve Asset-specific Work/Action Management Information

V3-0-8011-nnQueryAsRFW.xsd			(Returns Requests for Work related to an Asset)

V3-0-8025-nnQueryAsCompWork.xsd		(Returns Asset completed work history)

### Serve Segment-specific Work/Action Management Information

V3-0-8010-nnQuerySgRFW.xsd			(Returns Requests for Work related to a

Segment)

V3-0-8024-nnQuerySgCompWork.xsd		(Returns Segment completed work history)

### Serve General Work/Action Management Information

V3-0-8016-nnQueryWR.xsd			(Returns a Work Request)

V3-0-8023-nnQueryWOStepforWR.xsd		(Returns a Work Order Step for a Work

Request)

V3-0-8021-nnQueryWOStep.xsd	(Returns a Work Order Step)

V3-0-8019-nnQueryWO.xsd			(Returns a Work Order)

V3-0-8017-nnQueryWRAudit.xsd	(Returns the status of a Work Request)

V3-0-8022-nnQueryWOStepAudit.xsd	(Returns the status of a Work Order Step)

V3-0-8020-nnQueryWOAudit.xsd	(Returns the status of a Work Order)

### Receive New Asset Request for Work and Generate New Work Request

V3-0-8007-nnCreateAsRFWandWR.xsd	(Creates an Asset Request for Work (RFW) and a Work Request (WR) structure from a client-supplied RFW structure.  The returned Work Request primary key will be utilized by the client to track the work submitted.)

### Receive New Segment Request for Work and Generate a New Work Request

V3-0-8009-nnCreateSgRFWandWR.xsd	(Creates Segment Request for Work (RFW) & Work Request (WR) from a submitted client RFW.  The returned Work Request will be utilized by the client to track the work submitted.)

### Receive New Binary Attachment to Work Request

V3-0-8031-nnCreateWRBlob.xsd	(Creates a Binary attachment to a Work Request.  This interface can be repeated, if the server supports multiple binary attachments.  A server may or may not allow binary attachments.

### CRIS Entities Reference List

Work-XML Server, Read/Write Access, Support Level 1

Main Tables (28)      * referenced

*Enterprise

*Site

*Site_Database

DB_MIM_Interface

*Segment

*Asset

Solution_Package

Request

As_Req_For_Work

Sg_Req_For_Work

Work_Request

Work_Req_Blob_Data

Work_Request_Audit

Agent

As_Recommendation

Sg_Recommendation

As_Rec_Remark

Sg_Rec_Remark

As_Rec_As_Event

Sg_Rec_Sg_Event

Asset_Event

Segment_Event

Work_Order

Work_Order_Audit

Work_Order_Step

Work_Order_Step_Audit

Sg_Completed_Work

As_Completed_Work

Type Tables (18)

Priority_Type

Solution_Pack_Type

Agent_Type

Work_Manage_Type

Work_Task_Type

Blob_Data_Type

Blob_Content_Type

Eng_Unit_Type (if supported)

Eng_Unit_Enum (if supported)

Ref_Unit_Type (provided by MIMOSA)

Work_Audit_Type

MIM_Interface_Type (provided by MIMOSA)

MIM_Data_Cat_Type (provided by MIMOSA)

MIM_Support_Type (provided by MIMOSA)

MIM_Func_Type (provided by MIMOSA)

MIM_Tech_Type (provided by MIMOSA)

MIM_Access_Type (provided by MIMOSA)

Row_Status_Type (provided by MIMOSA)